Model Submission Checklist

| Checks | Notes |
| --- | --- |
| Project Level Checks |  |
| Geo-referencing Check that model has coordinates within the below range: Northing (or Y coordinates): 20,000m ~ 50,000m Easting (or X coordinates): 0m ~ 60,000m |  |
| Levels Check that levels with the same name in different linked files has the same FFL. |  |
| Site Boundary Check that Site boundary is exported as IfcGeographicElement. Site boundary should be exported as a mass instead of planes |  |
| Earthworks Both existing and proposed terrain are available, exported to IfcGeographicElement, Status = Existing or Proposed respectively. | Proposed terrain need not be represented as a single IfcGeographicalElement if the proposed terrain is represented by the proposed landscape and hardscape. |
| File Size File Size of each IFC model or Revit model should not exceed files size limit of 800mb for uploading into CorenetX. |  |
| Export issues flagged out by IFC viewers Elements not properly exported to IFC are flagged as unreferenced elements in BIM viewers (eg. BIMvision). | Installing the latest IFC exporter in Revit minimises such errors. |
| Parameter checks |  |
| Basic Parameters Check that the following Parameters were exported correctly: - AGF_Name - AST_AreaType - AGF_DevelopmentUse - AVF_IncludeAsGFA - ALS_LandscapeType - SGPset_AreaDimension or NetFloorArea |  |
| Hygiene Parameter check - Checks that certain parameters are declared correctly |  |
| All Private Strata areas are included as GFA Check that all IfcSpace with AST_AreaType containing “Private” has AVF_IsIncludeAsGFA = “Yes” |  |
| All Areas included as GFA should have Development Use declared Check that all IfcSpace with AVF_IsIncludeAsGFA = “Yes” has AGF_DevelopmentUse filled up. |  |
| Tabulation (Additional Checks) - Checks that the totals of parameters are correct. |  |
| GFA Check that the following matches the declared value: - Total GFA (AVF_IncludeAsGFA = Yes) - Total Bonus GFA (AVF_IncludeAsGFA = Yes, AGF_BonusGFAType is not empty) - GFA per development use (for mixed used development) (AVF_IncludeAsGFA = Yes, grouped by different AGF_DevelopmentUse) | Core information used for URA’s review of IFC models are: - AVF_IncludeAsGFA - AGF_BonusGFAType - AGF_DevelopmentUse - AGF_Name |
| No. of Dwelling Units No. of Dwelling units (identified using AGF_Name) with unique unit no. (identified using AGF_UnitNumber) matches declared value. |  |